Effect of wave spectral variability on the dynamic response of offshore wind turbine considering soil-pile-structure interaction

Shanghui Yang 1, Xiaowei Deng 1*, Mingming Zhang 2, Yixiang Xu 3

1. Department of Civil Engineering, The University of Hong Kong, Pokfulam, Hong Kong, China

2. School of Mechanical Engineering and Automation, Harbin Institute of Technology, Shenzhen 518055, China

3. School of Aerospace, The University of Nottingham Ningbo China, Ningbo 315000, China

(* Corresponding author: xwdeng@hku.hk )

Abstract

The accurate assessment of the dynamic response of the offshore wind turbine is fundamental to its economical and safe design, where the precise characterization of the incident wind and waves is essential. While reliable aerodynamic models can accurately reproduce wind characteristics, the widely-used empirical JONSWAP spectrum in wave field simulation may not capture the multi-modal feature of the mixed wave field near the wind farm. In this study, the stochastic structural responses based on an integrated FE model of the soil-pile-structure system are assessed employing the empirical and actual wave spectra, along with discussions on the impacts of wave theory and wave-structure interaction. The main conclusions are: (1) The empirical wave spectrum can overestimate the dynamic responses compared with the measured swell-dominant one, but the dominant wind loads weaken its influence on displacement. (2) When the wave serves as the decisive loading, using the empirical spectrum overestimates the dynamic performance for the swell-dominant wave spectrum while underestimating it for the wind-induced-wave-dominant one. The approximation of the actual wave spectrum to the JONSWAP spectrum is only recommended when the coupling effect of wind and wave is inapparent. (3) The wave theory imposes an evident effect on the dynamic behavior of the offshore wind turbine subject to joint wind and wave loading, where the JONSWAP wave spectrum is more sensitive. (4) The influence of water-monopile interaction is negligible when using the JONSWAP wave spectrum but prominent for the simulation’s actual wave spectrum.

Keywords: offshore wind turbine; stochastic response; wave spectrum; wave theory; wave-structure interaction.

1. Introduction

The rapid expansion of offshore wind power in the global energy market creates an urgent demand for an economical and safe design framework for the offshore wind turbine exposed to complex environmental conditions, where the accurate estimation of the structural performance of the offshore wind turbine serves as a significant prerequisite. Currently, most offshore wind energy is being exploited in shallow water, where the severe wind and sea states inevitably impose new challenges and uncertainties to the reliable prediction of the design load in the dynamic analysis of the offshore wind turbine [1]. Moreover, owing to the large slenderness ratio of the structure, the offshore wind turbine is very sensitive and vulnerable to the external dynamic vibrations mainly sourced from waves and wind, which may cause destructive damage and affect its serviceability [2]. The previous literature investigated the dynamic behavior of the offshore wind turbine and revealed that the accurate characterization of the turbine response relied on the appropriate modeling of the stochastic nature of the incoming wind and waves, as well as the interaction of the different physics concerning the aerodynamics, hydrodynamics, and soil dynamics, etc. [3, 4]. Although present reliable and mature aerodynamic models can reproduce the wind-induced stochastic loads with decent accuracy [5], some overly simplistic assumptions on the hydrodynamic properties of the wave pose the obstacle to the reliable prediction of the system response exposed to the rough sea state.

Along with the massive upscaling of the offshore wind farms, the more giant wind turbines will gradually take the place of the conventional low-watt one to accomplish the target of more economical and efficient power production [6]. However, the dynamics of the larger turbine structure will be increasingly interfaced with the more violent wave action, indicating the essential role of the wave in the assessment of structural safety and reliability. The wave spectrum describes the energy distribution of different frequency components and is often used for the stochastic simulation of sea surface elevation, which determines the critical wave characteristics. Therefore, the precise characterization of the stochastic nature of the wave field calls for a good understanding of the wave spectra in the shallow seas. In the existing literature, the stochastics wave is commonly generated based on some empirical unimodal wave spectra, mainly including Joint North Sea Wave Project (JONSWAP) [7] and Pierson & Moskowitz (PM) [8] wave spectra, targeted at the developing and fully developed sea states, respectively. Furthermore, according to DNV guidelines, the current engineering practice to model stochastic waves on offshore wind turbines is limited to JONSWAP wave spectrum. Nevertheless, as to the ocean environment favored for the offshore wind turbine, the swell driven by permanent and homogenous winds generally coexists with the wave generated by local wind, leading to the insufficiency of the unimodal description [9, 10]. Therefore, some advanced wave spectra models fully catering to the multi-modal feature of the aforementioned mixed wave field have been proposed, such as Ochi-Hubble spectrum [11], Guedes Soares spectrum [12], and Torsethaugen spectrum [13]. Notwithstanding, the wave spectra models mentioned above are all empirically deduced based on the measurements at the specific sea area, resulting in some inconsistencies with the actual wave spectrum owing to the lack of understanding of the complex air-sea interactions governing the wave growth and decay [14]. Meanwhile, due to complex coastal geography and seabed terrain in the nearshore area, the frequent occurrence of wave shoaling and refraction will result in the significant spatial variation of the wave heights and directions, namely, the spatially inhomogeneous wave field. Given the further deviation of stochastic system response induced by the aforementioned inaccurate grasp of the wave spectrum in the current research and engineering practice, it is necessary to investigate the effect of wave spectral variability on the dynamic response of the marine structure. Ti et al. [15] studied the impact of wave spectra variability on the system response of sea-crossing bridges using a frequency-domain approach and revealed the disparities of stochastic response resulting from the wave spectra. However, there are few studies on its effect on the dynamic behavior of the offshore wind turbine accounting for soil-pile-structure interaction.

Moreover, the present simulation strategy of the stochastic waves on the offshore wind turbine is confined primarily to the linear wave theory, albeit several advanced nonlinear wave models [16, 17][16, 17] available for the modeling of the irregular nonlinear wave in the finite-depth water for wave loading prediction, have been developed. Furthermore, in view of the complex coupling physics of wave-structure interaction, the relative movement of water particles to the vibrating monopile is generally approximated by the absolute movement. The impacts of the aforementioned simplistic strategies on the accurate assessment of the structural performance of the offshore wind turbine have been widely discussed in previous literature [3, 18-21], while there are few studies on the effect of the wave spectral shape on the feasibility of these simplifications.

This paper focuses on the effect of wave spectral variability on the dynamic behavior of the offshore wind turbine under joint wind and wave loading. A 3D finite element model considering soil-pile-structure interaction is developed using ABAQUS to analyze the time-domain and frequency-domain characteristics of its stochastic responses in terms of the displacement and shear force at the mudline. Moreover, a parametric analysis is performed to further discuss the reliability of the aforementioned simplistic strategies in the dynamic simulation of the offshore wind turbine, serving as the guidance on the efficient and reasonable assessment of the structural dynamic response under joint wind and wave loading.

2. Wave spectra

A small part of the sea area in the China East Sea, where an offshore wind farm is under the plan, is selected as a benchmark example. As shown in Figure 1(a), the target sea region is dotted with islands, along with undulated seabed topography, and all wind turbines are mainly deployed in A, B, and C three regions. Figure 1(b) further provides information on the layout of the under-plan wind farm, where the wind turbines deployed in A and B points are selected for the subsequent investigation of the effect of the spatially inhomogeneous wave field. The measured wave spectrum within the same sea area in the literature [15] is also included in this paper to serve as a reference for the general wave spectrum pattern in this area. The seabed with detailed high-resolution bathymetry data and distribution of three points of the study are demonstrated in Figure 2.

(a)

(b)

Fig. 1. Basic information on target wind farm: (a) sea area of study and wind farm location; (b) layout of the under-plan wind farm.

Fig. 2. Water depth for the entire computational domain and three points of study.

As mentioned previously, the wave spectra variability is of paramount significance for the dynamic performance of offshore structures under stochastic loadings. In this study, three representative wave spectra in the target sea area mentioned above are selected to investigate the effect of wave spectral variability on the dynamic response of offshore wind turbines. The first is an actual wave spectrum obtained through spectral analysis of the measured wave elevations based on an ultrasonic wave gauge deployed in the measurement location (Figure 2) near the wind farm. The other two comes from the inhomogeneous wave field prediction of the target sea area inside the wind farm using the spectral wave model, see points A and B in Figure 2. In the meanwhile, their respective corresponding empirical wave spectrum with the same significant wave height and peak wave period are also acquired for comparison. JONSWAP spectrum, as the employed empirical wave spectrum in this study, is one of the most commonly-used wave spectra in the current wave engineering practice. It extends from the PM spectrum and has a more accurate grasp of the stochastic features of the developing sea state in severe conditions. It has the following form [7]:

(1)

(2)

Where f denotes wave frequency, fp is the peak wave frequency, fp = 1/Tp.  represents the peak-shape parameter, taking 3.3 in this study. C( is the normalizing factor,  is the spectral width, taking 0.07 when f≤fp and 0.09 when f>fp. In the following section, Hs and fp in Eq. (1) take the same value as the measured and simulation spectra for comparison purposes.

2.1 Measured wave spectra

The measured wave spectrum near the wind farm is shown in Figure 3, and the empirical JONSWAP wave spectrum with the same significant wave heights Hs and peak wave periods Tp is also displayed for comparison purposes. The actual wave elevation was measured by an ultrasonic wave gauge deployed near the wind farm at 119.621°E, 25.715°N, with ±0.2 m and ±0.25 s for wave height and period error, respectively. 2 Hz is selected as the sampling frequency, fully capturing most wave frequency components. Then the actual wave spectrum can be obtained through spectral analysis based on the time series of measured wave elevations. As can be seen from the figure, dual peaks occur in the measured wave spectrum, including a major low-frequency one and a minor high-frequency one. According to the discussions in previous literature, the former comes from the swell propagating from the open sea, while the latter is generated by the local wind. Furthermore, due to the relatively flat terrain in this region, the peak energy of the former is evidently higher than the one of the latter. The peak wave periods Tp can be directly identified from the first crest in the wave spectra as10.89 s, and the significant wave heights Hs can be further calculated as 2.67 m. Nevertheless, it can be seen from the figure that JONSWAP spectrum has a more pronounced single peak with the same Tp and Hs as the actual wave spectrum, 10.89 s and 2.67 m, respectively. By comparing measured and empirical wave spectra, it is clear that the empirical spectrum omits the mixed wave characteristic of the actual one but overestimates the low-frequency wave energy at the primary peak.

Fig. 3. JONSWAP and measured wave spectrum.

2.2 Simulation wave spectra based on spatially inhomogeneous wave field

In this case, the target sea area of the wind farm is full of complex slopes and ridges, as illustrated in Figure 2, which strongly affects ocean waves propagating from deep waters and induces a spatially inhomogeneous wave field. The measured wave spectrum mentioned above only exhibits a general double-peak feature of the actual wave spectrum near the wind farm, thus calling for a comprehensive grasp of the wave spectrum in the exact wind farm region, especially for its inhomogeneous wave field on complex terrain [22, 23]. Given the complex wave transformation in this area, a spectral wave model is constructed with high-resolution bathymetry data and the inlet waves in deep waters come from the northeast boundary [24]. The numerical model with the instructed mesh, illustrated in Figure 1(a), can take multiple wave physics into account, including wind-wave generation and wave transformation, and is capable of reproducing the wave field driven by inlet waves on the boundary and local wind. More relevant model details can refer to in the literature [15]. The output information of the numerical wave model includes wave spectrum and the corresponding wave characteristics, such as wave height, period, etc.

The significant wave height field by the spectral wave model is displayed in Figure 4(a), and the wave spectra at two typical points A at 119.778°E, 25.695°N and B at 119.659°E, 25.678°N within the wind farm zone are also extracted from the model outputs as shown in Figures 4(b) and 4(c). As for the wave spectrum at point A, a similar energy distribution trend to the measured one can be observed, except for less pronounced energy discrepancies for two peaks, indicating relatively comparable energy from the swell and local wind. This phenomenon can be explained by the fact that the region possesses a relatively gentle terrain with no obvious barrier but is a little closer to the lagoon surrounded by the shallow reefs and tiny islands than the measurement location. However, an opposite tendency displays in the wave spectrum at point B, where energy associated with the local wind wave surpasses the one pertinent to the swell, resulting in the peak wave period obtained from the high-frequency crest. The above phenomenon can attribute to the shallow reefs and tiny islands near position B, which block the wave propagation from the open sea like a barrier and thus result in the decay of the swell energy. Table 1 lists the significant wave height and peak wave period at positions A and B, respectively.

(a)

(b)

(c)

Fig. 4. Wave characteristics in wind farm: (a) significant wave height field in wind farm; (b) simulation wave spectrum at point A; (c) simulation wave spectrum at point B.

Table. 1. Wave characteristics within the wind farm.

| Point | Coordinate (LONG/LAT) | Significant wave height (m) | Peak wave period (s) |
| --- | --- | --- | --- |
| A | (119.778°E, 25.695°N) | 6.97 | 12.58 |
| B | (119.659°E, 25.678°N) | 5.28 | 5.78 |

3. Example offshore wind turbine and modelling description

In this study, a 5 MW wind turbine situated in the selected wind farm was taken as the example turbine, representing the conventional utility-scale offshore wind turbines with a variable-speed, collective pitch-controlled operating system. Table 2 provides a summary of the main turbine characteristics. The rotor-nacelle system is installed at the top of the tower with 334.8 tons in weight and 126 m in length. The tower has a height of 73 m with the linear variation of diameter and wall thickness, 4.25 m to 6 m and 0.04 m to 0.08 m, respectively. The total length of the pile is 67.57 m, where 35.5 m was embedded in the soil, with the same diameter and wall thickness as the tower base. The tower and pile have the same mechanical properties, with Young's modulus of 210 GPa and Poisson ratio of 0.3. Considering the realistic geological conditions in the East China Sea, the soil profile parameters are listed in Table 3.

Table. 2. Parameter of a representative 5-MW offshore wind turbine in the East China Sea.

| Parameter |  |
| --- | --- |
| Tower height (m) | 73 |
| Tower top diameter (m) | 4.25 |
| Tower bottom diameter (m) | 6 |
| Tower wall thickness (m) | 0.04 |
| Tower mass (kg) | 170300 |
| Young's modulus of tower material (GPa) | 210 |
| Top mass (kg) | 334800 |
| Monopile diameter (m) | 6 |
| Monopile wall thickness (m) | 0.08 |
| Young's modulus of monopile (GPa) | 210 |
| Embedded length of monopile (m) | 35.5 |

Table. 3. Parameter of the soil profile.

| Parameter | Finite element zone | Infinite element zone |
| --- | --- | --- |
| Effective unit weight se (kN/m3) | 10 | 10 |
| Young's Modulus E (MPa) | 40 | 40 |
| Poisson's ratio | 0.25 | 0.25 |
| Internal friction angle deg) | 35 | / |
| Dilation angle (deg) | 5 | / |
| Cohesion c (kPa) | 0.1 | / |

The simulation work in this study is mainly conducted on the software ABAQUS, where a 3D FE model of the integrated soil-pile-structure system was constructed and a series of dynamic analyses was further performed based on the aforementioned model. As shown in Figure 5, the FE model is comprised of two main components, turbine and foundation soil, where the former can be further divided into rotor and nacelle, tower and monopile, and the latter include fill soil inside the pile and outer soil outside the pile. As for the turbine, different element types are selected for the three parts, where the rotor and nacelle are simplified as a point mass at the tower top, and the tower and monopile are both modeled by the shell element with a linear elastic behavior. For the soil foundation, the brick element is adopted for both fill and outer soil, except for the infinite element at the outermost edges to circumvent the artificial boundary effect on the dynamic performance of the turbine [25]. It should be noted that the fill soil is assumed to fully plug the penetration depth of the pile [26] and the outer soil has a 10 D diameter and 1.6 L depth. The bottom of the outer soil domain is fixed against horizontal translation, while no extra constraint on the lateral boundary due to the employment of the infinite element. The mesh size of soil and pile remains 1 m based on the result of mesh sensitivity analysis in reference [27], the same as spring spacing in the following distributed spring model. It is worth noting that the gradual mesh refinement is applied to the pile region subjected to wave loading owing to the substantial variation of wave loading near the water surface. The complex material behaviour of the soil is characterized by the elastic-perfectly plastic Mohr-Coulomb constitutive model, ABAQUS built-in small-sliding, surface-to-surface master/slave contact pair formulation is adopted to model the soil-pile interaction, specifically, penalty method in the tangential direction and the Lagrange multiplier method in the normal direction [28]. Compared with the conventional p-y curve-based foundation model, 3D continuum modelling of the soil using the above finite element method (FEM) is deemed the most accurate and flexible foundation modelling approach [29]. On the one hand, the various constitutive models can reflect soil behaviours more realistically than the p-y curve; on the other hand, the continuum effect of stress distribution along the pile can be captured accurately. Current innovations in this foundation modelling approach mainly involve novel soil-pile contact algorithms and high-cyclic soil models [30, 31], exhibiting decent advantage in modelling the long-term performance of the offshore wind turbine whereas bringing high computational costs. However, the main objective of this study is to analyze the effect of wave spectral variability on the short-term dynamic performance of offshore wind turbines. The employment of the simple Mohr-Coulomb constitutive model can aid in the simulation of the short-term soil-pile interaction with acceptable accuracy and also improve computational efficiency compared with complicated soil constitutive models for cyclic loading. More innovations on the foundation modelling approach will be implemented for the research on the long-term performance of offshore wind turbines, accounting for more complex soil behaviour like strain accumulation under cyclic loading, and serve as an independent study.

One key issue to be addressed ahead of the dynamic analysis of the offshore wind turbine lies in the reasonable estimation of the vibration damping, which generally includes aerodynamic damping, structural damping, hydrodynamic damping, and soil damping. As for the soil damping resulting from the soil-pile interaction, the wave-radiation damping part is negligible considering the main low-frequency component of wave and wind loading, and only material damping is accounted for in this study. Referring to the damping estimation in the previous research, we select 6.21% as the damping ratio of the fore-aft direction [20, 32, 33], which then helps determine the mass and stiffness coefficients based on Rayleigh damping.

Fig. 5. 3D FE model.

The fundamental frequency of the structure is one of the key indicators grasping the dynamic behaviour of the offshore wind turbine, which can serve as the initial verification of the numerical credibility and efficacy of the aforementioned FE model. Three types of real-world offshore turbines accounting for the representative turbine size and surrounding soil condition are extracted from the literature, and the detailed structural data and foundation information are summarized in Table 4 [34]. As shown in the table, the monopile diameter covers a range from 3.2 m to 6 m; simultaneously, the discrepancies between the sand and clay are also considered to examine the FE model's generality under different site conditions.

Table. 4. Structural data of OWT.

| Offshore wind turbine | Walney 1 | Lely A2 | North Hoyle |
| --- | --- | --- | --- |
| Tower height (m) | 67.3 | 37.9 | 67 |
| Tower top diameter (m) | 3 | 1.9 | 2.3 |
| Tower bottom diameter (m) | 5 | 3.2 | 4 |
| Tower wall thickness (mm) | 40 | 13 | 35 |
| Young's modulus of tower material (GPa) | 210 | 210 | 210 |
| Top mass (t) | 234.5 | 32 | 100 |
| Monopile diameter (m) | 6 | 3.2 | 4 |
| Monopile wall thickness (mm) | 80 | 35 | 50 |
| Young's modulus of monopile material (GPa) | 210 | 210 | 210 |
| Monopile depth (m) | 23.5 | 13.5 | 33 |
| Shear modulus of the soil (MPa) | 70 | 140 | 230 |
| Poisson's ratio of the soil | 0.4 | 0.4 | 0.4 |
| Young's modulus of the soil (MPa) | 196 | 392 | 644 |

The FE models of the selected offshore wind turbines are established based on the data in Table 4, and the corresponding natural frequencies are obtained through Eigen-frequency analysis. Moreover, the site investigation provides the measured natural frequency as the reference for comparison purposes. Meanwhile, the results based on the well-validated distributed spring model recommended by the API [35] and DNV [36] are also offered for further comparison. In the distributed spring model approach, the soil-pile interactions are characterized by a series of lateral and vertical springs, where the lateral spring (p-y curve) represents the lateral resistance, while the vertical springs symbolize the shaft friction (t-z curve) and the end bearing capacity at the pile end (Q-z curve), respectively. The primary nonlinear lateral resistance-deflection relationship in the distributed spring model, namely, the API p-y curve, follows the expression:

(3)

where p denotes the lateral soil resistance, kN/m, y is lateral deflection, m, k represents the initial modulus, kN/m3, determined based on the internal friction angle , z is the depth, m, A is a constant, taking 0.9 for cyclic loading, pu is the ultimate strength, defined as pus and pud for the shallow and deep depths, respectively:

(4)

where  denotes the effective soil weight, D denotes the pile diameter, C1, C2, and C3 are coefficients determined by the internal friction angle . Comparing the measured and predicted data in Table 5, they are in good agreement regardless of turbine size and soil conditions, among which the prediction accuracy of the FE model surpasses the one based on the distributed spring model slightly. To sum up, the above comparisons preliminarily lend credence to applying the FE model to subsequent dynamic analysis of the offshore wind turbine under joint wind and wave actions. Further validation of the dynamics response compared with the commonly-used distributed spring model will also be performed in Section 5.1.

Table. 5. Site condition and fundamental frequency of OWT.

| Offshore wind turbine | Walney 1 | Lely A2 | North Hoyle |
| --- | --- | --- | --- |
| Soil condition at the site | sand layers | sand and clay layers | sand and clay layers |
| Measured fundamental frequency | 0.35 Hz | 0.64 Hz | 0.35 Hz |
| Predicted fundamental frequency (FE model of the soil-pile system) | 0.31 Hz | 0.70 Hz | 0.34 Hz |
| Predicted fundamental frequency (Distributed spring model) | 0.31 Hz | 0.71 Hz | 0.37 Hz |

4. Aerodynamic and hydrodynamic model

The accurate prediction of the stochastic dynamic response of the offshore wind turbine under external vibrations plays an essential role in ensuring its structural safety and reliability during the whole lifetime. As the two primary external vibration sources, the wind and wave loads on the offshore wind turbine are stochastically generated in accordance with the sophisticated techniques recommended by DNV and then the dynamic simulations in the time domain under joint actions of wind and wave are carried out to capture the dynamic behaviour of the overall structures.

4.1 Aerodynamic model

4.1.1 Stochastic wind

The wind velocity can be decomposed into a constant mean wind velocity and a fluctuating wind velocity, where the vertical profile of the former is subject to [37]:

(5)

where  is the Von Karman constant, taking as 0.4, z0 denotes roughness height, set to 0.005, u* is the friction velocity determined based on the mean wind velocity at the hub height u = 7 m/s.

As for the stochastic simulation of the fluctuating wind velocity along the tower, the spectral representation method [38, 39] is adopted in this study, where the problem is regarded as a 1D multivariate stochastic process { f1(t), f2(t), …, fn(t)} obeying:

(6)

where N is a large number symbolizing the number of spectral segments, up is the upper cut-off frequency, the frequency bandwidth ∆up/N), Φ1l,…, Φjl (l = 1, …, N) are a series of random sequences satisfying uniform distribution in [0, 2π], H() can be obtained through the Cholesky’s decomposition of the cross-spectral density matrix S0():

(7)

and

(8)

where ∆jm and Coh(∆jm, represents the distance and the coherence function between positions j and m, respectively, the latter is described by the following function:

(9)

where  represents the co-coherence decay coefficient.

The Kaimal spectrum in Figure 6 is adopted to describe the power spectral density (PSD) distribution of the fluctuating wind velocity, which can be expressed as follows [40]:

(10)

Fig. 6. Kaimal wind spectrum.

4.1.2 Wind loads

The wind loading on the offshore wind turbine includes two components based on the force acting positions, one is the loading on the blades and the other is the loading on the tower. In this study, we only take the parked condition of the offshore wind turbine into account. As for the actions on the blades, the blade element momentum (BEM) method [41] is employed to determine the thrust force, where the blade is discretized into N cells and the total resultant forces can be calculated through the integration along the blade diameter. With regard to each cell, the force vector can be decomposed into lift force FL and drag force FD as follows:

(11)

where CL and CD are the lift coefficient and drag coefficient, c and ∆r represent the chord length and the radial length of the blade element, Vrel demotes the relative wind velocity using the following equation:

(12)

where Ω represents the angular speed of the rotor, Vx and V denote the axial velocity and tangential velocity on the blades, r is the radial distance from the nacelle center to the cell center. As shown in Figure 7, the thrust force Fx in the axial direction for each blade element can be calculated by:

(13)

where  denotes the relative wind angle, determined by:

(14)

where  represents the attack angle, and p and t are the blade pitch angle and twist angle, respectively.

Fig. 7. Forces on each blade element.

A far as the loading on the tower is concerned, the force at the specific height z can be calculated as:

(15)

where Cdt represents the drag coefficient, taking 1.2, A(z) is the frontal area at location z, , air is the air density, taking 1.225 kg/m3, u is the total wind velocity at position z.

4.2 Hydrodynamic model

4.2.1 Stochastic wave

The current model characterizing stochastic wave mainly deals with the linear wave theory, where  can be expressed as the superposition of different frequency contents:

(16)

Where wm represents the mth frequency segment, Am is the Rayleigh distributed amplitudes subject to E[Am2] = 2S(wm)Δw, m is the random phase satisfying a uniform distribution in [0, 2π]. The corresponding horizontal velocity u and acceleration a of water particles at depth z can be calculated based on the following functions:

(17)

(18)

Where km denotes the wave number of the mth frequency segment and h represents water depth. However, due to the shallow water depth at wind farm sites, waves generally show pronounced nonlinearity, which requires an accurate representation of nonlinear wave nature. This study introduces a second-order nonlinear irregular wave model proposed by Sharma and Dean [17], which assumes wave as a sum of first- and second-order components. The first-order component has the same form as the linear wave theory, while the second-order component takes the interactions of sums and differences of frequencies into account and the detailed formulation of wave height is as follows:

(19)

(20)

Where 1 and 2 are the wave heights of the first- and second-order components, m = wmt+m, Bmn+ and Bmn- are second-order transfer functions, based on the solution of Laplace's equation for velocity potential considering nonlinear B.Cs. Similarly, the velocity potential can be given:

(21)

(22)

(23)

Where bm = Amg/wm and kmn±= |km±kn|, Dmn+ and Dmn- are second-order transfer functions, which are also obtained from Laplace’s equation for velocity potential. According to the definition of velocity potential, the velocity u and acceleration a of water particles in the horizontal direction at depth z are given as follows:

(24)

(25)

4.2.2 Wave loads

Once the wave kinematics information is given, the wave loading on the pile foundation can be determined using the Morison equation. Referring to DNV, the wave force per unit length at depth z [36, 42] can be calculated as:

(26)

where D is the diameter of monopile, is water density, CM denotes the inertia coefficient. q and q are the velocity and acceleration of structure in the horizontal direction at depth z, while u and a are the corresponding velocity and acceleration of water particles at the same position, which is relevant to the sea surface elevation . For the simplification of the calculation, some studies neglected water-monopile interaction and the corresponding wave force per unit length can be simplified as:

(27)

5. Results and discussion

5.1 Dynamic responses based on the measured wave spectrum

This section will unfold a deep discussion on the discrepancies of the dynamic behaviour resulting from the measured and empirical wave spectra based on the proposed FE model mentioned above. The time series of the dynamic response, including the horizontal displacement and the shear force at the mudline, are extracted from the numerical simulation and then the further frequency-domain and statistical analysis are performed simultaneously. In the realistic operating phase of the offshore wind turbine, it bears stochastic wind and wave loading simultaneously. Therefore, the external vibration induced by the joint wind and wave actions is both incorporated in the subsequent dynamic simulation to investigate the response discrepancies based on measured and empirical JONSWAP spectra under joint wind and wave action. Section 3 has provided the preliminary validation of the proposed FE model in terms of the fundamental frequency based on the measurements. Considering the lack of measured data for the dynamic response of the target turbine, the simulation data based on the well-validated distributed spring model recommended by the API and DNV, as shown in Figure 8, will serve as the reference to verify the efficacy of the proposed FE model in the dynamic analysis. The distributions of p-y springs, t-z springs, and Q-z springs are illustrated in Figure 8(a), which are all modelled using the ground spring element in ABAQUS, with a spacing of 1 m. The related parameter in the p-y curve is listed in Table 6, and the p-y curves at different depths are also shown in Figure 9.

| (a) | (b) |
| --- | --- |

Fig. 8. Different foundation models: (a) distributed spring model; (b) FE model soil-pile system.

Table. 6. Parameters of p-y curve.

| Parameter |  |
| --- | --- |
| Effective unit weight se (kN/m3) | 10 |
| C1 | 3 |
| C2 | 3.48 |
| C3 | 54.75 |
| k (kN/m3) | 2274.14 |

Fig. 9. p-y curves.

Table 7 list the maximum and RMS of the response, respectively. Obviously, the results using the FE model are close to the ones based on the distributed spring model concerning the displacement and internal force response, further verifying the reliability of the FE model in simulating soil-pile-structure interaction. Furthermore, as mentioned in the reference [19], the proposed FE model is superior to the distributed spring model in considering the effects of soil densification and numerous loading cycles.

Table. 7. Structural responses compared with the distributed spring model.

| Foundation model | umax (cm) | urms (cm) | Fmax (kN) | Frms (kN) |
| --- | --- | --- | --- | --- |
| Distributed spring model | 3.559 | 1.914 | 102.111 | 46.397 |
| FE model of soil-pile system | 3.455 | 1.910 | 108.283 | 49.045 |

The subsequent discussions will concentrate on the investigation of the effect of wave spectral variability on the dynamic behavior of the offshore wind turbine. Figure 10 shows the PSDs of the displacement and shear force at the mudline. As for the displacement response in Figure 10(a), there are also two evident peaks, but the first peak appears near the peak frequency of the wind spectrum along with the same position of the second peak close to the natural frequency of the offshore wind turbine. Moreover, the crest related to the peak frequency of the wave spectrum disappears, and the high-frequency domain near the natural frequency contains the higher concentrated energy. When it comes to the difference of response energy distribution between the two wave spectra, it can be observed that the displacement response related to the empirical spectrum has slightly higher energy near the peak frequency of the wave spectra and the natural frequency. As far as the shear force response at the mudline is concerned in Figure 10(b), three peaks co-occur, which correspond to the peak frequency of the wave and wave spectra, and the fundamental frequency of the structure, respectively, with the most dominant one relevant to the wave spectrum. Sharp distinctions of the peak energy between the internal force response concerning the two wave spectra can be seen, especially for the one near the peak frequency of the wave spectrum, while the relatively modest difference at about the natural frequency. Overall, the shear force response based on the JONSWAP spectrum always contains higher energy than the one based on the measured spectrum, except for the component near the secondary peak frequency of the measured wave spectrum.

(a)

(b)

Fig. 10. PSDs of the structural response under joint loadings: (a) displacement at the tower top; (b) shear force at the mudline.

To make a further quantitative comparison, the time histories of dynamic responses are illustrated in Figure 11. Meanwhile, the corresponding RMS and maximum of the time-series response data mentioned above are calculated and listed in Table 8. It is clear that the wave spectrum difference almost imposes an insignificant impact on the displacement response unlike the discussion in section 4.1.1, implying that the wind loading plays a more pivotal role in the displacement response of the offshore wind turbine than the wave loading. When it involves the shear force response, a more considerable decrease is experienced in the case of the measured wave spectrum, reaching up to 16% for the difference of the maximum response. To sum up, owing to the dominant role of the wind loading on the displacement response at the tower top, the influence of the wave spectrum can be ignored. Nevertheless, when it comes to the internal force response, the disparities resulting from the empirical and measured wave spectra should be attached importance to, especially for the shear force at the mudline. In more detail, the aforementioned dynamic response in the case of the empirical wave spectrum can be overestimated, especially for the low-frequency concentrated energy near the peak frequency of the primary crest of the wave spectrum.

(a)

(b)

Fig. 11. Time histories of structural responses by using JONSWAP and measured spectra: (a) displacement at the tower top; (b) shear force at the mudline.

Table. 8. Structural responses by using JONSWAP and measured spectra.

| Wave spectrum | umax (cm) | urms (cm) | Fmax (kN) | Frms (kN) |
| --- | --- | --- | --- | --- |
| JONSWAP | 3.455 | 1.910 | 108.283 | 49.045 |
| Measured | 3.463 | 1.908 | 90.031 | 47.688 |
| Difference | 0.232% | -0.105% | -16.856% | -2.767% |

5.2 Dynamic responses based on the inhomogeneous wave field

To shed light on the influence of spectral disparities induced by inhomogeneous wave field on the dynamic performance of the offshore wind turbine under joint wave and wind loading, similar dynamic simulation considering the stochastic wave loading based on the simulation wave spectra at points A and B within the wind farm are carried out, accompanied by the counterparts based on JONSWAP spectrum for comparison.

5.2.1 Swell-dominant wave spectrum

As illustrated in Figure 4(b) of Section 2.2, more comparable double crests than the measured one can be observed in the simulation actual wave spectrum at point A. The swell energy still plays a prominent role but with a higher significant wave height. Two comparative dynamic simulations based on the simulation actual spectrum at point A and the counterpart JONSWAP spectrum are conducted to understand better the response difference resulting from the spectral variability mentioned above. Figure 12 demonstrates the obtained frequency characteristics of the corresponding displacement and internal force response. Unlike the energy distribution in Figure 10, the first crest occurs at the peak frequency of the wave spectrum instead of the wind spectrum, indicating the more pronounced impact of the wave loading on the displacement response. Moreover, more substantial differences exist between the displacement response based on the empirical and simulation spectra, especially for the two peak energies at the peak frequency of the wave spectrum and the natural frequency, respectively. Higher energy concentrates at about the natural frequency of the structure for the simulation spectrum, whereas the primary crest shifts to the peak frequency of the wave spectrum for the JONSWAP spectrum. On the other hand, it’s also worth noting that although a minor crest appears at the secondary peak frequency of the simulation wave spectrum, the displacement response enlarges the insignificant peak disparities shown in the wave spectrum. As far as the internal force is concerned, only one primary crest occurs near the peak frequency of the wave spectrum, and a minor peak still appears close to the secondary peak of the simulation spectrum for the specific case. Similarly, the peak discrepancies are evident for the internal force response based on the JONSWAP and simulation wave spectra.

(a)

(b)

Fig. 12. PSDs of the structural response using swell-dominant wave spectrum: (a) displacement at the tower top; (b) shear force at the mudline.

Figure 13 demonstrates the time histories of structural responses by using JONSWAP and swell-dominant spectra, and the direct quantitative comparisons of the time-domain responses are listed in Table 9, including the RMS and maximum. Unlike the pivotal role of wind loading in the dynamic performance for the case using the measured spectrum, the wave loading takes the wind's place and significantly impacts both the displacement and shear force response for the case utilizing the simulation spectrum at point A. Similarly, the empirical spectrum increases the kinematic and kinetic responses remarkably, especially for the shear force response up to 40%. Furthermore, its influence on the internal force exhibits a higher degree than the displacement response. To sum up, when it comes to using the swell-dominant wave spectrum with a relatively dominant role of the wave loading in dynamic behavior, the simplistic approximation of the empirical JONSWAP spectrum somewhat overestimates the dynamic performance in terms of the displacement and shear force. Moreover, the internal force response experiences a more considerable variation than the movement, where the energy discrepancies mainly concentrate on the low-frequency components near the primary peak frequency of the wave spectra.

(a)

(b)

Fig. 13. Time histories of structural responses by using JONSWAP and swell-dominant spectra: (a) displacement at the tower top; (b) shear force at the mudline.

Table. 9. Structural responses by using JONSWAP and swell-dominant wave spectra.

| Wave spectrum | umax (cm) | urms (cm) | Fmax (kN) | Frms (kN) |
| --- | --- | --- | --- | --- |
| JONSWAP | 5.319 | 2.247 | 628.451 | 241.011 |
| Simulation | 5.277 | 2.117 | 594.579 | 138.174 |
| Difference | -0.80% | -5.79% | -5.39% | -42.67% |

5.2.2 Wind-induced-wave-dominant wave spectrum

As mentioned above, the inhomogeneous wave field resulting from the complex terrain within the wind farm also brings about spectral pattern variability as well as fluctuations of some wave characteristics. To gain further insight into the influence of the wave spectral variability based on the diversified wave pattern, the simulation wave spectrum at point B is also picked up for the comparison of the dynamic response under the empirical and actual wave spectra. Owing to the right shift of the primary peak for the simulation wave spectrum, the peak energy of the corresponding JONSWAP spectrum appears near the relatively high-frequency range, namely, the peak frequency of the local wind wave. According to the PSDs of the displacement response shown in Figure 14(a), the aforementioned variation of the energy distribution for the simulation wave spectrum at point B leads to the weakening of the wave influence on the displacement response, where the primary and secondary crests appear near the natural frequency and the peak frequency of wind, respectively. An additional minor peak can be observed at about the swell frequency for the simulation spectrum, whereas the wave effect is intangible for the JONSWAP spectrum. In the light of the spectral distribution of the internal force response in Figure 14(b), there appear to be two comparable peaks pertinent to the peak frequency of the wave and natural frequency of the structure under the JONSWAP spectrum, respectively, while three peaks, including an extra primary peak near the swell frequency, co-exist under the simulation spectrum. Another noteworthy phenomenon is the opposite peak tendency of the internal force response between the swell frequency and wind wave frequency in contrast with the original wave spectrum.

(a)

(b)

Fig. 14. PSDs of the structural response using wind-induced-wave-dominant wave spectrum: (a) displacement at the tower top; (b) shear force at the mudline.

Considering a more direct view of the statistical characteristics using the RMS and maximum of time-domain data, the following sections only demonstrate the comparisons of the above quantitative indexes. Table 10 presents the RMS and maximum of the displacement and shear force response, respectively, to compare the time-series data quantitatively. In general, the empirical JONSWAP spectrum underestimates the displacement and shear force responses compared with the wind-induced-wave-dominant wave spectrum, showing an opposite trend to the swell-dominant wave spectrum. More specifically, the decrease of displacement is negligible in comparison with the shear force, consistent with the spectral energy difference shown in Figure 14. Therefore, contrary to the swell-dominant wave spectrum, the conversion of the primary peak for the wind-induced-wave-dominant wave spectrum can lead to the underestimation of the kinetic and kinematic responses based on the empirical JONSWAP spectrum. Similarly, the internal force response sees a sharper fall compared with the displacement response, which can be attributed to the additional, prominent peak near the swell frequency of the actual spectrum.

Table. 10. Structural responses by using JONSWAP and wind-induced-wave-dominant wave spectra.

| Wave spectrum | umax (cm) | urms (cm) | Fmax (kN) | Frms (kN) |
| --- | --- | --- | --- | --- |
| JONSWAP | 3.440 | 1.900 | 109.148 | 47.430 |
| Simulation | 3.460 | 1.920 | 132.000 | 52.700 |
| Difference | 0.58% | 1.05% | 20.90% | 11.10% |

In summary, the single-peak feature of the JONSWAP spectrum is incapable of capturing the coupling effect of wind and wave, especially when the swell dominates. Furthermore, the ignorance of the wind-wave component by the JONSWAP spectrum can produce a response estimation error of up to 6% in displacement and 42% in internal force with the gradual enhancement of the wind-wave interaction. When the energy of the wind wave rises to outperform the one of swell, the inconsideration of the swell component by the JONSWAP spectrum in this situation can also induce an error of 1% and 11% for the displacement and shear force prediction. Hence, the approximation of the JONSWAP spectrum to the actual simulation spectrum is only recommended when the coupling effect of wind and wave is unobvious. In contrast, the multimodal feature of the whole simulation spectrum needs to be emphasized when the wind-wave interaction gets stronger gradually.

5.3 Parametric studies

Considering different simplification techniques employed in the dynamic simulation of the offshore wind turbine, a parametric sensitivity analysis is conducted in this section to discuss their impact on the dynamic behaviour of the structure, especially for the cases employing the empirical JONSWAP and actual wave spectrum. Given the simplifications as mentioned above deal more with the wave loading, the simulation actual wave spectrum at point A is selected in the subsequent discussion, serving as a representative swell-dominant wave spectrum with a prominent significant wave height. The following discussion covers the critical issues on wave theory and water-monopile interaction.

5.3.1 Effect of wave theory

As mentioned before, most of the current wind farms are sited in shallow water, where the water wave shows nonlinear nature owing to water depth and terrain [43]. Nevertheless, the current engineering practice is generally confined to the linear wave theory in modeling the stochastic irregular wave in the dynamic simulation of the offshore wind turbine, leading to the inaccurate characterization of the realistic wave kinematics in shallow water. To investigate the influence of the wave theory, a set of dynamic simulations are performed using the empirical JONSWAP and simulation wave spectra. Figure 15 and 16 demonstrate the PSDs of the displacement response for two wave theories, respectively. Obviously, the significant difference can be found no matter for using JONSWAP and actual wave spectra, indicating that the simplification of the linear wave theory can be deemed unreasonable concerning the analysis of the displacement at the tower top. Moreover, the contrary impacts of nonlinear wave theory are imposed to the peak energy near the natural frequency under JONSWAP and actual wave spectra, respectively, with a marked rise for using the JONSWAP spectrum and a modest drop for using the simulation wave spectrum. In addition, nonlinear wave theory more or less diminishes the 2nd peak energy associated with the wind wave when using the simulation wave spectrum, as shown in Figure 15. As for the PSDs of the shear force for two wave theories in Figure 16, two PSD distributions almost overlap no matter for using the empirical and actual wave spectra, except for a slight difference close to the high-frequency energy zone with an opposite tendency for employing the JONSWAP and simulation wave spectra like the displacement response.

(a)

(b)

Fig. 15. Displacement PSDs for two wave spectra: (a) JONSWAP wave spectrum; (b) simulation wave spectrum.

(a)

(b)

Fig. 16. Shear force PSDs for two wave spectra: (a) JONSWAP wave spectrum; (b) simulation wave spectrum.

The detailed quantitative comparisons of the RMS of the time-domain response are illustrated in Table 11 and 12, in terms of the displacement and shear force response. Consistent with the above PSD distributions' variation, the RMS of displacement response shows an opposite trend for two wave theories when using the JONSWAP and actual wave spectra. A notable rise in the displacement for the nonlinear wave theory appears when using the JONSWAP spectrum but a modest decrease for the nonlinear wave theory when using the actual wave spectrum. Moreover, a higher degree of variation is manifested using the JONSWAP wave spectrum. When it comes to the spectral characteristics of the shear force response, a similar trend to the displacement response can be observed for the nonlinear wave theory under the JONSWAP and actual wave spectra. Besides, the shear force response is sensitive to the wave theory to almost the same degree as the displacement. On the whole, the wave theory imposes an evident effect on the dynamic behavior of the offshore wind turbine subject to joint wind and wave loading. Specifically, the nonlinear wave theory can bring about a noted increase of the displacement and shear force response under the JONSWAP wave spectrum, but a modest decrease under the simulation wave spectrum, implying contrary influences of second-order wave component for the JONSWAP and actual wave spectra on the dynamic performance of the offshore wind turbine. Moreover, the JONSWAP wave spectrum is more sensitive to wave theory than the simulation actual wave spectrum.

Table. 11. RMS of displacement response.

| Wave theory | JONSWAP wave spectrum | Simulation wave spectrum |
| --- | --- | --- |
| Linear wave theory | 2.247 cm | 2.117 cm |
| Nonlinear wave theory | 2.354 cm | 2.095 cm |
| Difference | 4.76% | -1.04% |

Table. 12. RMS of shear force response.

| Wave theory | JONSWAP wave spectrum | Simulation wave spectrum |
| --- | --- | --- |
| Linear wave theory | 241.011 kN | 138.174 kN |
| Nonlinear wave theory | 249.620 kN | 136.482 kN |
| Difference | 3.57% | -1.22% |

5.3.2 Effect of wave-structure interaction

Apart from the accurate capture of the wave kinematic characteristics, another critical issue on the stochastic simulation of the wave loading lies in the reasonable calculation of the wave loading using the Morison equation as mentioned before. According to Eq. (26), the water-monopile interaction is incorporated in the item of relative movement of water particles to the monopile. In the realistic wave engineering practice, considering the complexity of real-time feedback of the monopile movement, the relative movement of water particles is generally approximated by the absolute movement. The dynamic simulations considering the wave loading calculation based on the relative movement and absolute movement are carried out to explore the feasibility and reliability of the above approximation method. Figure 17 shows the PSDs of the displacement response for the two calculation approaches to wave loading, respectively. The PSD distributions considering absolute and relative movement almost overlap when using the JONSWAP spectrum, whereas there is a remarkable energy disparity between ignoring and including water-monopile interaction when using the simulation actual wave spectrum, especially for the suppression of the crest near the natural frequency structure when accounting for the water-monopile interaction. The PSDs distribution of the shear force response in Figure 18 displays a similar tendency, where the effect of the approximation method is negligible for using the JONSWAP spectrum, but its weakening effect on the peak energy around the high-frequency range under the simulation actual wave spectrum can be observed.

(a)

(b)

Fig. 17. Displacement PSDs for two wave spectra: (a) JONSWAP wave spectrum; (b) simulation wave spectrum.

(a)

(b)

Fig. 18. Shear force PSDs for two wave spectra: (a) JONSWAP wave spectrum; (b) simulation wave spectrum.

Table 13 and 14 provide the RMS of the time-series analysis of the displacement and shear force response, respectively, for the further quantitative comparison. The displacement response only sees a tiny decrease for ignoring water-monopile interaction under the JONSWAP wave spectrum, but a significant one when using the simulation actual wave spectrum, which is commensurate with the PSDs distribution. As for the shear force response, there appears to be a more marked drop for the approximation method under the simulation actual wave spectrum, but still a trivial difference exhibits under the JONSWAP wave spectrum. Generally speaking, no noticeable impact of the approximation method on the dynamic performance occurs when using the JONSWAP wave spectrum. In contrast, it can overestimate the dynamic response under the simulation actual wave spectrum compared with the accurate reference, especially for the internal force prediction.

Table. 13. RMS of displacement response.

| Wave-monopile interaction | JONSWAP wave spectrum | Simulation wave spectrum |
| --- | --- | --- |
| Ignoring interaction | 2.253 cm | 2.243 cm |
| Including interaction | 2.247 cm | 2.117 cm |
| Difference | -0.27% | -5.62% |

Table. 14. RMS of shear force response.

| Wave-monopile interaction | JONSWAP wave spectrum | Simulation wave spectrum |
| --- | --- | --- |
| Ignoring interaction | 241.779 kN | 164.738 kN |
| Including interaction | 241.011 kN | 138.174 kN |
| Difference | -0.32% | -16.12% |

6. Conclusions

The accurate assessment of the dynamic response of the offshore wind turbine exposed to complex environmental conditions is a significant prerequisite for the economical and safe design, where the appropriate modelling of the stochastic nature of the incident wind and waves plays an essential role. This paper conducts numerical studies on the dynamic performance of the offshore wind turbine exposed to severe wind and sea states, considering the stochastic wave loading simulation based on the empirical wave spectrum, the measured wave spectrum, and the simulation actual wave spectrum induced by the inhomogeneous wave field. A 5MW offshore wind turbine case study considering the soil-pile-structure interaction is performed to systematically analyze the influence of wave spectral shape and then comprehensively investigate its sensitivity to wave theory and wave-structure interaction. The main results are summarized as follows:

(a) Using the empirical wave spectrum can overestimate the dynamic responses compared with the measured swell-dominant wave spectrum. However, the dominant role of wind loading can weaken the influence of wave spectral variability on the displacement response, but the disparities in internal force response should still be attached to importance.

(b) When the wave acts as the decisive loading, the influence of wave spectra variability exhibits an opposite trend under the swell-dominant and wind-induced-wave-dominant spectra. Specifically, using the JONSWAP spectrum overestimates the former's dynamic performance while underestimating the latter. The approximation of the actual wave spectrum to the JONSWAP spectrum is only recommended when the coupling effect of wind and wave is unobvious.

(c) The nonlinear wave theory can bring about a notable increase in the displacement and shear force response under the JONSWAP wave spectrum but a modest decrease under the simulation wave spectrum. Moreover, the JONSWAP wave spectrum is more sensitive to wave theory than the simulation actual wave spectrum.

(d) Ignoring the water-monopile interaction does not affect the dynamic performance when using the JONSWAP wave spectrum. In contrast, it can overestimate the dynamic response under the simulation actual wave spectrum, especially for the internal force prediction.

(e) The influence of nonlinear irregular waves on the displacement and internal force response is evident, especially when using the JONSWAP wave spectrum. However, the effect of the water-monopile interaction is negligible for using the JONSWAP wave spectrum but should be taken seriously under the simulation actual wave spectrum.

It is worth noting that the actual wave spectrum employed in this study is specific to the East China Sea; thus, more wave spectra samples in the favored location for the offshore wind farm need to be collected to analyze the generality of the above conclusions. Moreover, the present study is only targeted at the short-term dynamic performance of the offshore wind turbine, thus calling for a further investigation of the effect of wave spectral variability on the long-term dynamic behavior exposed to cyclic wind and wave loading in future studies. Meanwhile, the current FE model considering soil-pile-structure interaction will not be applicable to the fatigue analysis of the offshore wind turbine based on long-term cyclic loading. Therefore, a more complex soil model for cyclic loading will also be incorporated into the basic FE model to feed into the in-deep investigations on fatigue lifetime prediction and structure optimization of the offshore wind turbine. In addition, some critical factors like the foundation property and operational condition will also be accounted for in the future fatigue analysis of the offshore wind turbine.

# Acknowledgment

This work is generously supported by the Research Grants Council via RGC/CRF (C7038-20GF) and RGC/GRF (17204122), Shenzhen Science and Technology Innovation Committee via Fundamental Research General Program (2021302011), and National Natural Science Foundation of China via General Program (Project No. 51878586).

Reference:

[1]	 W. Musial, S. Butterfield, and B. Ram, "Energy from offshore wind," in Offshore technology conference, 2006: OnePetro.

[2]	S. Bhattacharya, "Challenges in design of foundations for offshore wind turbines," Eng. Technol. Ref, vol. 1, no. 1, p. 922, 2014.

[3]	P. Agarwal and L. Manuel, "Incorporating irregular nonlinear waves in coupled simulation and reliability studies of offshore wind turbines," Applied Ocean Research, vol. 33, no. 3, pp. 215-227, 2011.

[4]	M. Damgaard, M. Bayat, L. V. Andersen, and L. B. Ibsen, "Assessment of the dynamic behaviour of saturated soil subjected to cyclic loading from offshore monopile wind turbine foundations," Computers and Geotechnics, vol. 61, pp. 116-126, 2014.

[5]	J. M. Jonkman and M. L. Buhl, FAST user's guide. National Renewable Energy Laboratory Golden, CO, USA, 2005.

[6]	V. Igwemezie, A. Mehmanparast, and A. Kolios, "Current trend in offshore wind energy sector and material requirements for fatigue resistance improvement in large wind turbine support structures–A review," Renewable and Sustainable Energy Reviews, vol. 101, pp. 181-196, 2019.

[7]	K. Hasselmann et al., "Measurements of wind-wave growth and swell decay during the Joint North Sea Wave Project (JONSWAP)," Ergaenzungsheft zur Deutschen Hydrographischen Zeitschrift, Reihe A, 1973.

[8]	W. J. Pierson Jr and L. Moskowitz, "A proposed spectral form for fully developed wind seas based on the similarity theory of SA Kitaigorodskii," Journal of geophysical research, vol. 69, no. 24, pp. 5181-5190, 1964.

[9]	K. Ewans, H. F. Hansen, and A. R. Zeeberg, "A spectral description for extreme sea states offshore Denmark Part I: Power spectrum," Ocean Engineering, vol. 246, p. 110351, 2022.

[10]	M. Anjali Nair and V. Sanil Kumar, "Wave spectral shapes in the coastal waters based on measured data off Karwar on the western coast of India," Ocean Science, vol. 13, no. 3, pp. 365-378, 2017.

[11]	M. K. Ochi and E. N. Hubble, "Six-parameter wave spectra," in Coastal Engineering 1976, 1977, pp. 301-328.

[12]	C. G. Soares, "Representation of double-peaked sea wave spectra," Ocean Engineering, vol. 11, no. 2, pp. 185-207, 1984.

[13]	K. Torsethaugen, "A two peak wave spectrum model," 1993.

[14]	L. Cavaleri, B. Fox-Kemper, and M. Hemer, "Wind waves in the coupled climate system," Bulletin of the American Meteorological Society, vol. 93, no. 11, pp. 1651-1661, 2012.

[15]	Z. Ti, K. Wei, Y. Li, and B. Xu, "Effect of wave spectral variability on stochastic response of a long-span bridge subjected to random waves during tropical cyclones," Journal of Bridge Engineering, vol. 25, no. 1, p. 04019131, 2020.

[16]	E. Marino, C. Borri, and U. Peil, "A fully nonlinear wave model to account for breaking wave impact loads on offshore wind turbines," Journal of Wind Engineering and Industrial Aerodynamics, vol. 99, no. 4, pp. 483-490, 2011.

[17]	J. N. Sharma, DEVELOPMENT AND EVALUATION OF A PROCEDURE FOR SIMULATING A RANDOM DIRECTIONAL SECOND-ORDER SEA SURFACE AND ASSOCIATED WAVE FORCES. University of Delaware, 1979.

[18]	E. Marino, C. Lugni, and C. Borri, "A novel numerical strategy for the simulation of irregular nonlinear waves and their effects on the dynamic response of offshore wind turbines," Computer Methods in Applied Mechanics and Engineering, vol. 255, pp. 275-288, 2013.

[19]	S. Yang, X. Deng, and J. Yang, "Modeling of soil-pile-structure interaction for dynamic response of standalone wind turbines," Renewable Energy, vol. 186, pp. 394-410, 2022.

[20]	H. Zuo, K. Bi, and H. Hao, "Dynamic analyses of operating offshore wind turbines including soil-structure interaction," Engineering Structures, vol. 157, pp. 42-62, 2018.

[21]	E. Marino, C. Lugni, and C. Borri, "The role of the nonlinear wave kinematics on the global responses of an OWT in parked and operating conditions," Journal of Wind Engineering and Industrial Aerodynamics, vol. 123, pp. 363-376, 2013.

[22]	V. Aboobacker, P. Vethamony, S. Samiksha, R. Rashmi, and K. Jyoti, "Wave transformation and attenuation along the west coast of India: Measurements and numerical simulations," Coastal Engineering Journal, vol. 55, no. 1, pp. 1350001-1-1350001-21, 2013.

[23]	T. Zilong and D. X. Wei, "Layout optimization of offshore wind farm considering spatially inhomogeneous wave loads," Applied Energy, vol. 306, p. 117947, 2022.

[24]	N. Booij, R. C. Ris, and L. H. Holthuijsen, "A third‐generation wave model for coastal regions: 1. Model description and validation," Journal of geophysical research: Oceans, vol. 104, no. C4, pp. 7649-7666, 1999.

[25]	N. Strömblad, "Modeling of soil and structure interaction subsea," 2014.

[26]	K. Paik, R. Salgado, J. Lee, and B. Kim, "Behavior of open-and closed-ended piles driven into sands," Journal of Geotechnical and Geoenvironmental Engineering, vol. 129, no. 4, pp. 296-306, 2003.

[27]	 A. Haiderali and G. Madabhushi, "Three-dimensional finite element modelling of monopiles for offshore wind turbines," in Proceedings of the world congress on advances in civil, environmental, and materials research, Seoul, 2012, pp. 3277-3295.

[28]	H. Ma, J. Yang, and L. Chen, "Numerical analysis of the long-term performance of offshore wind turbines supported by monopiles," Ocean engineering, vol. 136, pp. 94-105, 2017.

[29]	A. M. Page, K. S. Skau, H. P. Jostad, and G. R. Eiksund, "A new foundation model for integrated analyses of monopile-based offshore wind turbines," Energy Procedia, vol. 137, pp. 100-107, 2017.

[30]	H. P. Jostad, B. M. Dahl, A. Page, N. Sivasithamparam, and H. Sturm, "Evaluation of soil models for improved design of offshore wind turbine foundations in dense sand," Géotechnique, vol. 70, no. 8, pp. 682-699, 2020.

[31]	G. Nikitas, N. J. Vimalan, and S. Bhattacharya, "An innovative cyclic loading device to study long term performance of offshore wind turbines," Soil Dynamics and Earthquake Engineering, vol. 82, pp. 154-160, 2016.

[32]	C. Chen and P. Duffour, "Modelling damping sources in monopile‐supported offshore wind turbines," Wind Energy, vol. 21, no. 11, pp. 1121-1140, 2018.

[33]	S. Bisoi and S. Haldar, "Dynamic analysis of offshore wind turbine in clay considering soil–monopile–tower interaction," Soil Dynamics and Earthquake Engineering, vol. 63, pp. 19-35, 2014.

[34]	D. A. Bouzid, S. Bhattacharya, and L. Otsmane, "Assessment of natural frequency of installed offshore wind turbines using nonlinear finite element model considering soil-monopile interaction," Journal of Rock Mechanics and Geotechnical Engineering, vol. 10, no. 2, pp. 333-346, 2018.

[35]	A. P. Institute, Recommended Practice for Planning, Designing, and Constructing Fixed Offshore Platforms-Working Stress Design: Upstream Segment. API Recommended Practice 2A-WSD (RP 2A-WSD): Errata and Supplement 1, December 2002. American Petroleum Institute, 2000.

[36]	D. N. Veritas, "Design of Offshore Wind Turbine Structure," Offshore Standard DNV-OS-J101, 2004.

[37]	M. van der Laan, N. N. Sørensen, P.-E. Réthoré, J. Mann, M. C. Kelly, and J. Schepers, "Nonlinear eddy viscosity models applied to wind turbine wakes," Proceedings for the ICOWES2013, Copenhagen, Denmark, pp. 514-525, 2013.

[38]	M. Shinozuka and G. Deodatis, "Simulation of stochastic processes by spectral representation," 1991.

[39]	M. Shinozuka and C.-M. Jan, "Digital simulation of random processes and its applications," Journal of sound and vibration, vol. 25, no. 1, pp. 111-128, 1972.

[40]	J. C. Kaimal, J. Wyngaard, Y. Izumi, and O. Coté, "Spectral characteristics of surface‐layer turbulence," Quarterly Journal of the Royal Meteorological Society, vol. 98, no. 417, pp. 563-589, 1972.

[41]	M. Hansen, Aerodynamics of wind turbines. Routledge, 2015.

[42]	A. P. Institute, Recommended practice for planning, designing, and constructing fixed offshore platforms. American Petroleum Institute, 1989.

[43]	 A. R. Henderson and M. B. Zaaijer, "Hydrodynamic loading on offshore wind turbines," in The Fourteenth International Offshore and Polar Engineering Conference, 2004: OnePetro.